How to stream a pattern image with VD55G0

- Disable 2 lps DUSTER CONTROL REGISTER

DUSTER_CTRL ADDRESS: 0x0316

DUSTER_CTRL VALUE: 0x00

- Disable DARKCAL CONTROL REGISTER (Bypass Darkcal average)

DARKCAL_CTRL ADDRESS: 0x032C

DARKCAL_CTRL VALUE: 0x02

- Enable pattern PATTERN GENERATION REGISTER and select pattern type

PATGEN_CTRL ADDRESS: 0x0400

PATGEN_CTRL VALUE: 0x01 enable pat + pattern type

- Once the image has been transmitted, restore the default values in the registers to allow image acquisitions.

PATGEN_CTRL DEFAULT VALUE: 0x00

DUSTER_CTRL DEFAULT VALUE: 0x13

DARKCAL_CTRL DEFAULT VALUE: 0x01